BOCARAM/2 PLUS UPDATE INFORMATION

Some of the information contained in the BOCARAM/2 PLUS manual has

changed since the first printing and these changes may affect the way

in which you install your board.

BR2PTEST.EXE             Protected mode memory test

INTRODUCTION

Included on this diskette is a program called BR2PTEST.EXE. This program tests

the memory on the BOCARAM/2 PLUS in protected mode, a special processor mode

that allows direct addressing of extended memory. Because the memory is

directly accessed, the memory is tested with faster cycle times, and

intermittent faults are detected more easily.

IMPORTANT: This test is designed to run under DOS 3.0 or better ONLY. Do not

attempt to run this test under the WINDOWS or OS/2 DOS compatibility boxes.

Also do not run this test when a 386 memory manager is loaded such as DOS 5.0

EEM368 or Quarterdeck's QEMM386. All of these applications can re-map memory in

such a way as to be undetectable by this test, and the results will not be

valid. IN NO WAY WILL BR2PTEST HARM THE SYSTEM THAT IS UNDER TEST, the worst

that can happen is false errors. For further information see the Technical

Information section later in this document.

SETTING UP THE TESTS

BR2PTEST presents an interface similar to the set configuration screen in the

IBM reference disk program used to install the BOCARAM/2 PLUS. It asks for the

user to select the amount of memory used on the BOCARAM/2 PLUS board, and the starting

address of the board. A sample screen follows:

������������������������������������������������������������������������������Ŀ

�                              Boca Research, Inc.                             �

�                         BOCARAM/2 PLUS Memory Tests                          �

�                                                                              �

�                                                                              �

�                                                                              �

�  Memory On BOCARAM/2 PLUS.................[ 2M (2 SIMM modules) ]            �

�  Starting Address.........................[ 1Meg plus 384K (1408K) ]         �

�  Test Status:                                                                �

�  Not Running                                                                 �

�                                                                              �

�                                                                              �

�                                                                              �

�                                                                              �

�                                                                              �

�                                                                              �

�                                                                              �

�                                                                              �

�                                                                              �

�                                                                              �

�                                                                              �

�                                                                              �

� F1=Help   F3=Exit   F5=Previous   F6=Next   F10=Start Tests                  �

� Use arrow keys to make selection...                                          �

��������������������������������������������������������������������������������

In the actual screen, one of the choice boxes (enclosed by the square brackets)

will be highlighted. This is the active choice box. To move the highlight to

the other choice box, use the up or down arrow keys.

To select the correct choice, press the F5 or F6 keys until the proper choice

appears.

For a short help screen, press F1.

To exit the program WITHOUT running any tests, press F3.

To start the tests running, press F10. Once the test has begun there is no

way to stop the test short of turning the system off and back on. A message

appears in place of the FKEY prompts indicating this. You may turn off the

system at any time without any damage.

To determine the correct choices for your system, insert the reference

diskette into your A: drive, reboot, and read the currently installed choices

from your view configuration screen. (This also will cause a temporary 164

system error. See your BOCARAM/2 PLUS manual to correct this.) Make a note of

the data, and when you run BR2PTEST, select the same data for the test.

RUNNING THE TESTS

To run the test, simply press the F10 key when the data is correct. The test

status 'Not Running' will be replaced by various types of memory tests such

as 'Walking Bit, Checkerboard, Random Data, Bit Stuck and Block Move'. The

meaning of these tests is explained in the Technical Information section.

These tests are attempting to exercise the memory to the limits of the system

speed, and are simulating actual use.

The format of the messages that appear when the tests are running are as

follows:

TEST TYPE at XX0000 to XXFFFF, test count = X

where TEST TYPE is one of the previously mention tests, XX0000 to XXFFFF is the

addresses of the current 64k under test, and test count is the number of loops

that the test has undergone. The ability to loop this test is an advanced

feature accessible only from command line switches, and will be discussed in

the Technical Information section.

To the right of the message is displayed a spinner that indicates activity.

This is your assurance that the test has not locked up. Should the screen

freeze, with no spinner activity, then there is a system error. A cause for

this could be a 386 memory manager. Refer to the Introduction to solve this

problem.

If the test completes with no errors, then the following message will be

displayed:

TEST COMPLETE, NO ERRORS REPORTED.

Should the test find any errors, then the following error message will be

displayed:

ERROR READING DATA AT address. READ = data1, EXPECTED = data2.

where 'address' is the address of the error, and the data are the cause of the

message. The tests will continue, with the error stored. The error message will

remain under the active test message until the end of the test. Then a box

will be displayed with up to 5 errors that has occurred during the course of

this test. If 5 errors occur, then the test will abort.

The error display looks like this:

������������������������������������������������������͸

� NUM �      TEST      �  ADDRESS  �  EXPECT �   READ  �

������������������������������������������������������͵

�     �                �           �         �         �

�     �                �           �         �         �

�     �                �           �         �         �

�     �                �           �         �         �

�     �                �           �         �         �

������������������������������������������������������;

The box will be filled in with the data that caused the error.

Some errors do not cause improper data but cause what is known as a protected

mode exception. For example, a parity error causes a NON-MASKABLE INTERRUPT.

Some of these possible errors are:

Divide by zero

Single step

Parity error (NMI) interrupt at...

Breakpoint

Overflow

Bounds exception

Invalid Opcode

80287/387 not present

Double exception error

80287/387 segment overrun

Invalid TSS

Segment not present

Stack segment overflow

General protection fault

Page Fault

All of these 'exception errors' cause the system to lock up except for the

parity error.  All of these errors will be displayed. Should you get any of

these errors except parity, suspect that a memory manager is loaded. Refer to

the introduction to solve these problems.

A parity error will report the address that caused the error, and resume the

tests.

TROUBLESHOOTING MEMORY ERRORS USING BR2PTEST

The only replaceable parts on the BOCARAM/2 PLUS are the memory modules

(commonly known as SIMMS). This section explains which SIMM to replace should

a faulty  SIMM be detected.

Each SIMM is capable of storing 1 Megabyte of information organized as 1 Meg

by 8 bits. Since this is a 16 bit board, the minimum population is 2 SIMMS, or

2 Megabytes. In order to orient you, it is suggested that you have a board or

at least a picture of one in front of you as you read this.

Orient the board so that the edge connector is toward you. This will place the

SIMM area to your left. The SIMMS are in two columns of four. When populating

the board, the upper row of two is used first. This is the first 2 Megs of

16 bit memory. The second row of SIMMS are the third and fourth megabytes of

memory. The third row down is the fifth and sixth megabytes of memory and the

bottom row are the seventh and eighth megabytes of memory.

The SIMMS are organized so that when the computer reads the memory, the low

(or even addressed) byte is the right hand column of SIMMS and the high byte

(or odd addressed) is the left hand column of SIMMS.

If BR2PTEST detects a memory error, and you wish to change the SIMM that is

detected to be bad, use the following method to located the particular SIMM.

Determine the starting address of the board. Convert the starting address to

a 24 bit HEX representation of the address. (1 Meg = 100000h) Subtract this

number from the address that appears in the error messages.  (Remember to

do the arithmetic in HEX!). This will yield an offset into the board.

For example, a number of PS/2's have 1Meg 384k on the mother board. To calculate

the starting address use the following method:

1Meg of ram equals 16 64k blocks.

16 64k blocks in HEX is 10h.

Divide 384k by 64 to yield 64k blocks

384/64=6 which is 6h

Add 10h to 6h = starting address= 160000h

If the error was at 486D12 HEX then the offset into the board is

486D12h

- 160000h

---------

326d12h

or the error occurred between the second and the fourth megabyte, so the

affected SIMM is one of the two in the second row.

After studying the error message where is displays the READ, EXPECTED data,

you can determine whether the bad data is located in the high byte or the low

byte. If the error is in the high byte the SIMM to change is the left hand one,

conversely, if the error is in the low byte, then change the right hand SIMM.

For example, suppose that you have a BOCARAM/2 PLUS in your system with

4 Megabytes of ram.  The starting address is 2 Meg 256k. The error message that

you get when you run BR2PTEST is:

ERROR READING DATA AT 534300. READ 34FF EXPECTED 3423

First calculate the starting address. 2 Meg 256K in 24 Bit HEX notation is

240000h.  Subtracting this from the error's address yields 2F4300h as a

board address.  This means that the error is located in the second row down

from the top.

If we examine the data, we find that the low byte is the on that is returning

the wrong information. Therefore, the bad SIMM in this case is the right hand

one.  The bad SIMM is the second one down from the top in the right hand column.

TECHNICAL INFORMATION

This section is provided for the technically minded user that may have

questions about this information. Generally, most users will not be interested

in this information.

BR2PTEST operates under the protected mode model of the 80286, 80386 and 80486

microprocessors. In doing so, it provides its own complete 'operating system'

in that all interrupts are re-mapped, and it establishes all protected memory

segments. In this manner, it is incompatible with any 80386 memory managers

that utilize paging as the memory management scheme. This defines OS/2 V2.0

(yet to be released at this time), DOS 5.0 EEM386.EXE, WINDOWS 3.0 EEM386.SYS

and Quarterdeck's QEEM386, among others. In general, any memory manager that

employs memory paging can be a problem to this test.

The reason for this incompatibility is this: The 80386 (and better) has the

ability to map physical memory to appear at any address. This ability is called

'page mapping' or 'paging' and is used to make applications think that they are

running in memory addresses different than the actual physical address of the

memory that they are in. In this manner, these memory managers can make a real

mode application (which is, by definition, limited to running under 1 meg) run

in any part of the physical memory. The paging is transparent to the program.

If the memory map is different than the physical memory when BR2PTEST is run,

then BR2PTEST will not test the correct memory. This will cause no false errors,

but if there are errors, the addresses reported will be incorrect.

COMMAND LINE MODE

BR2PTEST optionally accepts command line information that specifies the testing

parameters. This is provided in case there is a desire to make a batch file

that specifies your particular system, thereby avoiding having to set the

correct specifications each time the test is run. The command line mode will

jump straight to the test without stopping to wait for the specifications.

To see a help screen that details the command line operation, type BR2PTEST /?.

The following will appear:

����������#����������Ŀ

BR2PTEST ����������������������������������������������������������Ĵ

#          #                    �                     �

����ram�����                    ���#��� /X�����������Ĵ

WHERE:                                        �          �      �   �

'ram' is the amount of memory in meg,  �          �� opt��   �

limited to 2, 4 or 8             �                     �

'/X'  selects specific tests           �����������/?����������

'opt' test options where:

'a' = all tests

'w' = walking bit tests

'c' = checkerboard tests

'p' = pseudo-random tests

'l' = address low tests

'h' = address high tests

'b' = bit stuck tests

'm' = block move tests

'1' thru '9' = number of times each test repeats

'/?'  prints this screen (no tests are performed)

'/Axx' sets the starting address, where 'xx' is in 64K blocks,

and is in hex notation. (see ram parameter above.)

Must be greater than 1M (10h).

'ram' stands for a one digit number that represents the amount of memory in

megabytes. It can be 2, 4, or 8.

Optionally, the command line can specify the test profile, by using the '/X'

parameter. In this way some tests can be selected, while others can be left out.

Also, the tests can be repeated by adding a single digit number to this

parameter. (A word of caution, though. Using repeats greater than 5 can take

a very long time.)

The '/X' parameter is optional.

If specific tests are desired, be sure to select all of the tests that you

wish to run.  For example, to run all of the tests, the parameter is '/Xa'.

To run the walking bit, checkerboard, low address, bit stuck and block move

tests the parameter is '/Xwclbm'. To run the walking bit test 5 times, the

parameter is 'Xw5'.

The starting address can be adjusted by the '/Axx' parameter. 'xx' represents

a two digit HEX number that specifies the starting address in 64K blocks. For

example, a number of PS/2's have 1Meg 384k on the mother board. To calculate

the starting address use the following method:

1Meg of ram equals 16 64k blocks.

16 64k blocks in HEX is 10h.

Divide 384k by 64 to yield 64k blocks

384/64=6 which is 6h

Add 10h to 6h = starting address= 16h

As another example, some PS/2's have 2 meg 384k on the mother board. So using

the method above:

2Meg = 20h

384k =  6h

Starting address = 26h

When you specify the parameter, leave off the 'h' character. It is used here

to identify HEX digits, but is not needed to specify the starting address.

For example, to start the tests beginning at 2Meg 384k, the parameter would

be '/A26'.

As a full example, suppose your system has a BOCARAM/2 PLUS in it with 4 Meg of

ram (4 SIMMS). Its starting address is at 1 Meg 384k, and you want to use the

walking bit, checker board, low address and bit stuck tests. You want to repeat

the tests 3 times.  The command line for this is:

BR2PTEST 4 /Xwclb3 /A16

The full suite of tests for BR2PTEST are not used in the default tests.

This concludes the documentation file for the BOCARAM/2 PLUS.

CBC 2/19/92 v1.1